- F.V 3D Printer

| Minimal Technical Specification: |  |
| --- | --- |
| Technology: | Fused filament fabrication (FFF) |
| Print head: | Dual extrusion print head with an auto-nozzle lifting system and swappable print cores |
| Build volume: | XYZ: 215 x 215 x 200 mm (left or right nozzle only) XYZ: 197 x 215 x 200 mm (dual extrusion) |
| Filament diameter: | 2.85 mm |
| Layer resolution: | 0.25 mm nozzle: 150 - 60 micron 0.4 mm nozzle: 200 - 20 micron 0.8 mm nozzle: 600 - 20 micron |
| XYZ resolution: | 12.5, 12.5, 2.5 micron |
| Print head travel speed: | 30 - 300 mm/s |
| Build speed: | Up to 24 mm/s 3 |
| Build plate: | Heated glass build plate |
| Build plate temperature: | 20 - 100 °C |
| Build plate leveling: | Active leveling |
| Build plate heat time: | <4 min (from 20 to 60 °C) |
| Supported materials: | Optimized for: PLA, Tough PLA, ABS, Nylon, CPE, CPE+, PC, PP, TPU 95A, PVA, Breakaway (Also supports third-party materials) |
| Nozzle diameter: | 0.25 mm, 0.4 mm, 0.8 mm |
| Nozzle temperature: | 180 - 280 °C |
| Nozzle heat up time: | <2 min |
| Build plate heat up time: | < 4 min (from 20 to 60 °C) |
| Operating sound: | 50 dBA |
| Connectivity: | Wi-Fi, LAN, USB port |
| Physical dimensions: |  |
| Dimensions: | 342 x 380 x 389 mm 342 x 505 x 588 mm (with Bowden tube and spool holder) |
| Required Power input: | 100 - 240 VAC / 50 - 60 Hz |
| Operating ambient temperature: | 15 - 32 °C, 10 - 90% RH non-condensing |
| Non-operating temperature: | 0 - 32 °C |
| Supported OS: | MacOS, Windows, and Linux |
| File types: | Ultimaker Cura: STL, OBJ, X3D, 3MF, BMP, GIF, JPG, PNG Printable formats: G, GCODE, GCODE.gz, UFP |
| Power Supply: | 220V AC,50Hz |
| Warranty period: |  |
| Warranty period: | 12 months |

- F.V 3D Printer filaments

| Minimal Technical Specification: |  |
| --- | --- |
| Filament diameter: | 2.85 mm |
| Materials: | Nylon, PLA, ABS |