- F.V 3D Scanner

| Minimal Technical Specification: |  |
| --- | --- |
| Cameras: | 2 x 2.8 megapixel Colour camera USB 3.0 cameras, 12mm lenses |
| Scan Speed: | 1.2 seconds per scan |
| Field of View (FOV): Adjustable to scan objects of different shapes and sizes | Preset: 200mm, 400mm, 600mm diagonal |
| Resolution: |  |
| Average Points: | 2.6 million per scan |
| Average Polygons: | 5.2 million per scan |
| Point to Point Distance: | 200mm FOV: 0.124mm 400mm FOV: 0.248mm 600mm FOV: 0.372mm |
| Accuracy: | 200mm FOV: 40µm (0.0016") 400mm FOV: 65µm (0.0026") 600mm FOV: 85µm (0.0033") |
| Standoff: | 200mm FOV: 431mm 400mm FOV: 914mm 600mm FOV: 1219mm |
| Geometry Formats: | PLY, OBJ, STL, ASC, FBX, 3D3 |
| Computer Requirements: |  |
| Supported OS: | Windows 7 (64-bit)/Windows 10 (64-bit) Operating System |
| Required Power input: | 100 - 240 VAC / 50 - 60 Hz |
| Warranty period: |  |
| Warranty period: | 12 months |